# NURSERY, GARDEN CENTER, GREENHOUSE & PATIO

5150 Garrison St.    Arvada, CO  80002    303-424-7979

http://www.echters.com/

Intro to Ponds & Water Features

- Intro:

  - A Water Feature is any element, indoors or out, that makes use of water:

    - Ponds, Streams and Water Falls

    - Small container Water Gardens

    - Fountains

  - Ponds and water features enrich and enhance our lives, beautify yards, aid in relaxation, can increase property value, and capture our imagination.

  - A well-designed pond will become a complete ecosystem and require little maintenance for most of the year and “fit” into its surroundings.

  - Our paradigms for building ponds have changed significantly over the past 10 years.

  - Water features are an investment, much like an outdoor hot tub.

  - Ponds MUST be researched before creating.  Make sure customers have done research and sell them books.

  - The #1 regret of first time pond owners is that they built it too SMALL.  The larger the pond, the easier it will be to maintain.

  - Water features are FUN!

- Six elements of a pond are liner, recirculatory system, filtration, rocks, fish and plants.

  - Liner

    - Rigid preformed liners

      - Easy to use and install, usually small, can be limiting

      - Frequently a first pond

    - Bulk flexible liner

      - Limited only by imagination

      - Allow for additions or changes in the future

      - Easy to repair, should a leak develop

      - Available in by the foot in 15, 20 and 30 foot widths

      - Ponds should have at least 1 foot of overhang liner on each side

      - Cutting Pond Liner

i. 	      Cut the liner only AFTER it has been purchased

  - Liner tears easily, use the lexan cutting surface and never drag the liner

  - 20’ & 30’ liner requires two people and a cart to cut, move and load

  - Liner, pond hose, bulk filter, and used pumps are NON-RETURNABLE

    - Flexible preformed

      - Fits in a box

      - Combine features of both

      - Have limitations of both

  - Recirculatory System

    - Water Pump

      - Provide water circulation and should also aerate the water

      - Should recirculate the total volume each hour in ponds under 250 gallons

      - Pumps should run continuously to keep adequate oxygen levels

      - The larger the pond the lower the % of water must be recirculated each hour

      - Keep in mind that for each foot of height a large percentage of volume is lost

      - Can be located within the pond or filter system or inline.

      - Pumps should always be designed for pond use!

    - Output

      - Usually a waterfall or stream

      - Could also be a wetland system

      - Some outputs will also act as a filter (Biofalls & wetlands)

    - Air pump

      - Increases oxygen levels in water for fish and bacteria

      - Eliminates “mat” algae on surface

  - Filtration

    - Basic Filtration prevents foreign objects from entering pump

    - Mechanical Filtration

      - Removes physical particles from pond

      - Most common is combination of skimmer and filter pads

      - Can be located in or out of pond, best where easily accessible

    - Biofiltration

      - Provides media for beneficial bacteria to colonize

      - Removes organic waste from ponds with bacteria

      - Necessary for fish health

      - Within (Biofalls) or separate (Tetra) from main recirculatory system

      - Can be easily retrofitted

    - UV Filter

      - Utilizes UV light to kill algae and parasites

      - Creates crystal clear water

      - Kills parasites and other water bourn diseases (also kills beneficial bacteria)

      - Only needs to run during day when algae is growing

      - Bulb needs replacement each year

    - Wetland filter

      - Utilizes rock, plants and bacteria to filter naturally

      - Looks natural

      - Does not provide skimming filtration

      - Great secondary filter system

  - Rocks & Gravel

    - Protects and covers liner

    - Looks natural

    - Keeps pond cleaner by creating a place for beneficial bacteria

    - Vary sizes and shapes of rocks for best, most natural look

  - Fish (Tadpoles, Turtles and More)

    - Add beauty and natural element to pond

    - Requirements

      - 2 ft depth minimum

      - Adequate aeration, more when water is warm

      - Cover to protect from predators and the sun

      - Ice melter during winter for gas exchange

    - Introduction

      - Rule is total square footage of surface = total number of inches of fish

      - Begin with a few add some each season

      - Acclimate to temperature

    - Feeding

      - Pond itself should provide adequate food

      - Fish food is bonus and can be supplemented with lettuce, melon, Cheerios or tuna.

      - Feed as much as fish will eat in 5 minutes up to 7 times a day

      - Feed fish in a deep area of the pond to deter predators from feeding

    - Fish Identification

      - Fantail- round body with double tail

      - Comet- orange/red or white & orange/red

      - Black Moor- black, round body with buggy eyes

      - Shubunkin- blue/white/orange/black speckled

      - Algae eater- tiny catfish like

      - Koi- have barbles, hard to catch can be any color or combo

      - Butterfly Koi- much longer fins/tails & perpendicular pectoral fins

      - calico markings- speckled coloration on fantails

      - sarassa markings- red/orange & white coloration on fantails & comets

    - Catching Fish & More

      - Fill bag ¼ to 1/3 with water

      - Bring net up under fish, cup fish in wet hand and place in bag

      - Keep bag full of air, twist top and close with rubber band

      - Write up ticket for cashiers and give customer care sheet

      - Pick up tadpoles and snails and place in bag

      - Grab turtles around sides, place in small cardboard tray and put in paper bag

  - Plants

    - Natural and necessary element

    - Shade pond and compete with algae to keep water clear

    - Should blend into surrounding landscape

    - Can act as filters

    - Provide food, shelter, and even a place to spawn

    - Blooming plants usually need fertilizers each year

- Maintenance

  - The more you have to do the less time you can enjoy your pond

  - Skimmer nets and filter pads as needed, best to clean with pond water

  - Everything else once at spring startup and once before fall shutdown

  - Test kits to monitor pH, ammonia, nitrates, nitrites, chlorine, and salinity

- Chemicals

  - Problem solvers for unbalanced ponds, artificially & temporarily restores balance

  - Chlorine / Heavy Metal removers make water safe for fish and bacteria

  - Water Clarifiers clump particles together so filter can remove

  - pH adjusters

  - Ammo Lock binds up toxic ammonia until biofilter can remove

  - Stress Coat provides slime for fish to ward off parasites & infection

  - Algaecides to kill string and single cell algae (green water)

  - Fish treatments for fungi, bacteria, or parasites (best to separate)

  - Salt

  - Plant fertilizers

- Natural Solutions

  - Bacteria to break down fish waste (MicrobeLift PL or BioFilter Booster)

  - Bacteria to break down other organic waste (Sludge Away, AquaZyme, Spring / Summer Cleaner

  - Bacteria to kill mosquito larva (MicrobeLift BMC, Mosquito Dunks)

  - Green Clean to remove string algae from waterfalls and streams

  - Barley Straw & Extract prevents algae naturally